Symbols Involved

Here's a list of all symbols used in the calculations, along with their definitions, units, and values based on your setup:

- D_o: Outer diameter of the pushrod. Value: 0.3125 inches (since 5/16" = 0.3125"). This is the total outside diameter of the tubular pushrod.

- D_i: Inner diameter of the pushrod. Value: 0.1525 inches. Calculated as D_o minus twice the wall thickness (hollow tube): D_i = D_o - 2 × wall thickness = 0.3125 - 2 × 0.080 = 0.1525".

- I: Moment of inertia (also called second moment of area). Units: inches⁴ (in⁴). Value: Approximately 0.00045 in⁴. This measures the pushrod's resistance to bending; higher I means stiffer pushrod.

- E: Young's modulus (modulus of elasticity). Units: pounds per square inch (psi). Value: 29,000,000 psi (standard for 4130 or 4340 chromoly steel). This is a material property indicating how stiff the steel is under tension/compression.

- L: Effective length of the pushrod. Units: inches. Value: 8.3 inches. Longer L reduces buckling resistance.

- P_cr: Critical buckling load. Units: pounds (lbs). Value: Approximately 1,870 lbs. This is the theoretical maximum compressive force before buckling.

- π: Pi (mathematical constant). Value: Approximately 3.1416. Used in circular geometry and buckling formulas.

Pushrod Strength and Stiffness:

- Moment of Inertia (I): For a tubular pushrod:

- I = π64 (Do4 − Di4)

Where Do​ = 0.3125", Di = 0.3125 − 2×0.080 = 0.1525" (inner diameter)

I = π64  (0.31254 − 0.15254) ≈ 0.00045 in4

Critical Buckling Load

Using Euler’s Critical Buckling Formula:

Pcr = π2EIL2

Where E is the modulus of elasticity (for 4130 steel, = 29,000,000 psi) and L is the effective length (8.3”)

Pcr = π2 x 29,000,000 x 0.000458.32  = 1870lbs

This buckling load is significantly higher than the expected valve spring forces (400lbs open valve spring pressure), indicating the pushrod can handle the compressive loads